Technical requirements document (TRD)

For

Minuteman mINIMUM Essential Emergency Communication Network (MEECN) Program (MMP)

Upgrade

ATTACHMENT 4

Solicitation# FA8726-07-R-0018

28 June 2007

Table of Contents

Paragraph	Page

1.0 Overview	5

1.1 Terminal Threat	5

1.2 Terminal Architecture	5

1.3 Program Scope	5

2.0 applicable documents	6

3.0 Technical Requirements	9

3.1 Terminal Architecture	9

3.1.1 System Flexibility	9

3.1.2 Software Communications Architecture (SCA) Requirements	9

3.2 TERMINAL EXTERNAL INTERFACES	9

3.3 Modes of operation	9

3.4 Terminal functional and Performance requirements	10

3.4.1 Backwards Compatibility	10

3.4.2 Interoperability	10

3.4.2.1 Message Processing Time	10

3.4.3 Probability of Correct Message Receipt	10

3.4.4 Communications Services	10

3.4.5 Network Support	11

3.4.6 EHF/AEHF Satellite Communications	11

3.4.7 Contingency Operations	11

3.4.8 AEHF Operations	11

3.4.8.1 AEHF Order wire and Transparent Messages	11

3.4.8.2 Satellite Transitions	12

3.4.9 Image and Ephemeris Data Management	12

3.4.9.1 Initialization Parameters and Ephemeris	12

3.4.9.2 Mission Planning Element (MPE) Data	12

3.4.9.3 UFO/E/EE Image Data	12

3.4.9.4 Over-the-Air Data Distribution (OTADD)	12

3.4.10 Cryptographic Equipment	12

3.4.10.1 COMSEC/Transmission Security (TRANSEC) Fill Device	13

3.4.10.2 Over-The-Air Rekeying (OTAR)	13

3.4.10.3 Key Management	13

3.4.10.4 Cryptographic Algorithms for COMSEC and TRANSEC	13

3.4.11 Operator Control	14

3.4.12 VLF/LF Timing Requirement	14

3.4.13 Prime Power	14

3.4.14 Recovery Timing Requirements	14

3.4.15 HAC/RMPE Requirements	15

3.5 Reliability	15

3.5.1 Failure Modes	15

3.5.2 Mean Time Between Critical Failures (MTBCF)	15

3.5.3 Operational Availability	16

3.6 Maintainability	16

3.6.1 Fault Detection and Isolation	16

3.6.2 Mean Time to Repair (MTTR)	16

3.6.3 Special Support Equipment	16

3.7 Survivability	16

3.7.1 Emergency Operation	17

3.7.2 Surge Protection	17

3.8 TEMPEST	17

3.9 Electromagnetic Interference/Electromagnetic Compatibility (EMI/EMC)	17

3.10 Software	17

3.11 Human System Integration	17

3.11.1 Manpower and Personnel	17

3.11.2 Safety	17

3.11.3 Two-Person Weight Restriction	17

3.12 Environmental Conditions	17

3.12.1 Operational Environment	17

3.12.2 Non-Operational Environment	18

3.13 PHYSICAL requirements	18

3.13.1 Rack Mounting	18

3.14 Packaging, Handling, and Transportation	18

3.14.1 Packaging	18

3.14.2 Transportation	18

3.15 Required Certifications	18

3.15.1 NC2-ESI Certification	18

3.15.2 EAM Certification	18

3.15.3 Nuclear Surety	18

3.15.4 Information Assurance Certification	18

3.15.5 DIACAP Certification	18

3.15.6 TEMPEST	18

3.15.7 Reserved	19

4.0 Quality Assurance Provisions	19

4.1 Requirements Verification	19

4.1.1 Subsystem Verification Testing	19

4.2 Acceptance Testing	19

5.0 List of Acronyms and Abbreviations	19

APPENDIX A	22

6.0 Terminal Requirements from SR-3000, Appendix M, Table M-23, Tailored for the MMP Upgrade Terminal	22

List of Tables

Table	Page

Table 3-1   MMPU Cryptographic Algorithm Table................................................................13

# Overview

The mission of the MMP terminals is to provide a high-confidence, survivable link for reception of Emergency Action Messages (EAMs) from the President and Secretary of Defense (SECDEF), the Joint Chiefs of Staff (JCS), and the Commander, United States Strategic Command (CDR USSTRATCOM) to the Intercontinental Ballistic Missile (ICBM) Launch Control Centers (LCCs) as well as a survivable force report-back capability supporting the Nuclear Execution and Reporting Plan (NEREP) process and STRATCOM generated Force Direction and Force Management messages. Survivable communications are key elements of our deterrence strategy because potential adversaries must be convinced that launch orders will actually be received by the LCCs and our communications, both receive and transmit, will not be easily disrupted.

The MMP Upgrade program will replace the existing Extremely High Frequency (EHF) Low Data Rate (LDR) function of the existing MMP terminals with LDR and Extended Data Rate (XDR) functions capable of operating on any JCS geosynchronous EHF/AEHF MILSATCOM system.  The Advanced Extremely High Frequency (AEHF) satellite constellation is the second increment of the planned Department of Defense (DoD) implementation of JCS EHF communications systems which will provide a dramatic increase in throughput capability.  The upgraded MMP will be fielded to provide operational terminals, training, test, and maintenance beginning in 2010, to ensure MMP users take advantage of AEHF throughput enhancements.

The MMP Upgrade program intends to increase flexibility of the upgraded MMP products, by providing an architecture that can be modified or extended to incorporate technology and capability upgrades. To this end, the MMP Upgrade terminal has a requirement for Software Communications Architecture (SCA) compliance.

## Terminal Threat

The current and projected threats to US space systems are detailed in the Space Capstone Threat Assessment, NASIC/FCIM DOD-1574-0727-06, March 2006 (S//FGI//NF//MR); Information Operations Capstone Threat Assessment, 5th Edition, DI-1577-33-06, Volumes 1-8, 10-14 and 16, January 2006 (S//NF); Information Operations Capstone Threat Assessment, 4th Edition, DI-1577-31-05, Volume 14, March 2005 (S//NF); Chemical, Biological and Radiological Warfare Capstone Threat Assessment, DI-1650-83-06, June 2006 (S//FGI GBR//NF//MR) and Military Satellite Communications (MILSATCOM) System Threat Assessment Report (STAR), NASIC-1574-0367-05, March 2005 (S//NF//MR).  Threat agent numbers and capabilities are outlined in Defense Intelligence Agency’s (DIA) The Postulated Threat: to U.S. Nuclear Weapon Facilities and Other Selected Strategic Facilities, January 2003 (S//NF//X1) and Air Force Intelligence Analysis Agency (AFIAA's) Worldwide Asymmetric Threat to USAF Installations, Personnel and Resources, August 2005 (S//NF).

## Terminal Architecture

## Program Scope

The MMP Upgrade program will extend the operational capability of the legacy MMP terminals by adding the AEHF backward-compatible (BC) LDR and XDR functions. The MMP Upgrade EHF terminal will provide the modem/processor function for EHF/AEHF communications, the antenna/RF function, and the operator control function.

The MMP Upgrade EHF terminal may reuse legacy MMP terminal components; the concept is neither mandated nor precluded (e.g., the terminal may use retained legacy MMP EHF antenna and RF equipment.) Any retained legacy equipment will be considered part of the MMP Upgraded terminal, and be subject to performance requirements of this TRD. Any modified legacy equipment will be considered as new equipment, and be subject to appropriate performance verification.

The terminal operator control function will be collocated with the MMP terminal equipment in the LCC.  Requirements for interfacing to the HAC/RMPE in the Weapon System Control Console (WSCC) may require changes to the HAC/RMPE to meet MMP Upgrade system level requirements.   The HAC/RMPE is an integral part of the Minuteman III EHF/AEHF system and it is important that the MMP Upgrade seamlessly interface with HAC/RMPE for mission accomplishment.

Any legacy MMP terminals components not used in the MMP Upgrade EHF terminal will be removed as the upgraded capability is fielded.

# applicable documents

The following documents of the exact issue shown form a part of this TRD to the extent specified herein.  In the event of conflict between the referenced documents and the contents of this document, the contents of this document shall be considered a superseding requirement.

| Number | Title |
| --- | --- |
| AFI 63-125 | Nuclear Certification Program, 15 March 2004 |
| AFPD 91-1 | Nuclear Weapons and Systems Surety, 1 November 1999 |
| CJCSI 5119.01B | Charter for the Centralized Direction , Management, Operation, and Technical Support of the Nuclear Command, Control and Communication System, 19 July 2004 |
| CJCSI 6130.01C | 2003 CJCS Master Positioning, Navigation, and Timing Plan, 31 March 2003 |
| CJCSI 6140.01A | NAVSTAR Global Positioning System Selective Availability Anti-Spoofing Module Requirements, 31 March 2004 |
| CJCSI 6811.01A | Nuclear Command and Control System Technical Performance Criteria (U), 9 June 2000. (SECRET) |
| D25-92853-V1-C | Interface Control Higher Authority Communications Integration and Control and Rapid Message Processing Element (HAC/RMPE) to the EHF Terminal Programming and Timing/Electrical, 14 Dec 2004 |
| DoDD 8500.1 | Information Assurance (IA), 24 October 2002 |
| DoDI 8510.bb | Interim DoD Certification and Accreditation (C&A) Process Guidance, DIACAP, 6 July 2006 |
| DoDI 5200.40 | DOD Information Technology Security Certification and Accreditation Process (DITSCAP), 30 December 1997 |
|  | Capability Development Document for Minuteman Minimum Essential Emergency Communications Network (MEECN) Program (MMP) Upgrade, 20 April 2007 |
| DoDI 8500.2 | Information Assurance (IA) Implementation, 6 February 2003 |
| EAP CJCS, Volume VII | (U) Emergency Action Procedures (EAP) of the Chairman of the Joint Chiefs of Staff, Volume VII (S) |
| FED-STD-1015 | Telecommunications: Analog to Digital Conversion of Voice by 2,400 bit/second Linear Predictive Coding, 28 November, 1984 |
| ICBM STAR | ICBM System Threat Assessment Report, January 2002 |
| ILES ORD | Operational Requirements Document (ORD), AFSPC 005-95B-I/II, ICBM Launch Control Center (LCC) Extremely High Frequency (EHF) System (ILES) for the ICBM Weapon Systems, 23 November 1998 |
| ISO/IEC 15408-1, 2, 3 | Information Technology – Security Techniques – Evaluation Criteria for IT Security – Parts 1, 2, and 3, 2005 |
| K00000531 | Advanced Extremely High Frequency (AEHF) COMSEC/TRANSEC System (ACTS) Variable Performance Chipset (VPC) Subsystem Specification, Rev E., 7 November 2003 |
| K00000552 | Advanced Extremely High Frequency (AEHF) COMSEC/TRANSEC System (ACTS) Variable Performance Chipset (VPC) Interface Control Document (ICD), Rev D., 14 Oct 2002 |
| AEHF KMP ver 4 | Advanced Extremely High Frequency (AEHF) Key Management Plan. Draft currently in ICWG. Requirement is for latest version at time of contract award. ICWG vote imminent.. |
| KMSD ver 4 | Advanced Extremely High Frequency (AEHF) Key Management System Description (KMSD). Draft currently in ICWG. Requirement is for latest version at time of contract award. ICWG vote imminent.. |
|  | Unified INFOSEC Criteria for MMP Upgrade |
|  | Telecommunications Requirements Document (TSRD) for MMP Upgrade |
| KOI-18 | KOI-18 COMSEC Tape Reader |
| MIL-HDBK-781A | Reliability Test Methods, Plans, And Environments For Engineering Development Qualification, And Production, 1 April 1996 |
| MIL-STD-1472F(1) | Department of Defense Design Criteria Standard – Human Engineering, 5 December 2003 |
| MIL-STD-3005 | Analog-to-Digital Conversion of Voice by 2,400 bit/second Mixed Excitation Linear Prediction (MELP), 20 December 1999 |
| MSRC-5000SCA | Software Communications Architecture, FINAL, 15 May 2006, Version 2.2.2, inclusive of the following appendices and attachments: Appendix A (Glossary), Appendix B (AEP), Appendix C (IDL), Attachment 1 to Appendix C, Appendix D (Domain Profile), Attachment 1 to Appendix D (DTDs), and Attachment 2 to Appendix D (Common Properties). |
| ON477312 | (U) Data Transfer Device (DTD) Interface Specification, AN/CYZ-10, September 24, 1992 |
| PMW 146-S-002 | UHF Follow-On (UFO) Extremely High Frequency (EHF) Telemetry, Tracking and Command (TT&C) and Communications Space Package (SP) Specification, 15 February 1999; plus SCN 5, 29 May 1995; plus SCN 6, 5 December 1996 (excluding GBS Spot Beam Pointing Protocol), Rev D |
| S-133-09259 | System Segment Specification for EHF and VLF/LF Higher Authority Communications Systems, Rev. B through SCN 3, 17 July 2003 |
| S-133-128C | System Specification for Minuteman III, 5 November 2004 |
| SI-1135 | Milstar LDR Payload to Terminal Interface Control Drawing, Revision F, 9 February 2001 - SCNs/IRNs F1, F2, F3, F4, and F5, and MSTICWG Changes 249 dated 8 September 2005 and 252 dated 4 August 2005 |
| SI-3135, Volume 1 | Advanced EHF Payload To Terminal ICD Volume 1 (IF2-0196-01), Rev H, 17 December 2003 plus CRN 1300 AEHF ICWG Approved 17 May 2004, CRN 1372-2 dated 16 November 2004, CRN 1316 dated 31 August 2005, CRN 1319 31 August 2005, CRN 1612 dated 14 September 2005, and CRN 1938 dated 28 September 2006.- Volume 1 with appendices A-C (XDR), (SECRET), 29 July 2005 |
| SI-3135, Volume 3 | Advanced EHF Payload To Terminal ICD Volume 3 (IF2-0196-03), Rev E, 17 December 2003, CRN 1319 dated 31 August 2005, and CRN 1612 dated 14 September 2005.with appendices A-H (LDR), (SECRET), 29 July 2005 |
| SI-3145, Volume 2 | Mission Control Segment (MCS) Common Interface to Terminal Segment Interface Control Document (ICD), Rev. G dated 14 October 2004, (includes the following change packages: MCS-148, MCS-149, MCS-150r1, MCS-151, MCS-152r4, MCS-153r3, MCS-155r2, MCS-156r7, MCS-157r3, MCS-158, MCS-164r5, MCS-166, MCS-167), IRN 1 (includes the following change packages: MCS-169r3, MCS-170r4, MCS-171r7, MCS-172r2 and MCS-178r4) plus IRN 1 dated 15 September 2004 (CRN 1282), IRN 2 (includes MCS-180) dated 15 October 2004 (CRN 1283), IRN 3 dated 9 February 2005 (CRN 1374), and IRN 4 dated 16 December 2004 (CRN 1088-6), CRN 1372-2 dated 16 November 2004, CRN 1569 dated 11 Aug 2005, CRN-1331A, 23 August 2006; MT-205, MT-211, MT-213, MT-217, MT-219, MT-221 and MT-223, and draft CRN 1342 dated 7 Jul 2006. |
| SR-2300 | Terminal Segment Specification For The Milstar II Satellite Communications Program SR-2300, Revision C, 10 September 2003 |
| SR-3000 | AEHF System Specification (8579100), Revision M, 21 November 2002, including CRN1088-2 change (PERB date: 3 March 2004), SR-3000 SCN-M5, 21 November 2003, SR-3000 SCN-M7, 16 January 2004, CRN 1093 dated 13 July 2004, CRN 1226 PERB date 30 June 2004, CRN 1372-2 dated 16 November 2004. |
| SR-3000, Appendix M | AEHF System Specification, Appendix M (8579100) - Terminal Requirements (U), Revision N, 1 April 2004, including CRN1088-2 change (PERB date: 3 March 2004), SCN-N1 ECP-04-003, dated 3 March 2004, CRN 1093 dated 15 July 2004, CRN 1226 PERB dated 30 June 2004, and CRN 1028 dated 17 December 2003. |
| SR-3300 | Joint Terminal System Specification (JTSS) for the AEHF Satellite Communications Program, Rev C, 16 April 2004 |
| DOD-1574-0727-06 | Space Capstone Threat Assessment, March 2006 (S//FGI//NF//MR) |
| DI-1577-33-06, Volumes 1-8, 10-14 and 16 | Information Operations Capstone Threat Assessment, 5th Edition, January 2006 (S//NF) |
| DI-1577-31-05, Volume 14 | Information Operations Capstone Threat Assessment, 4th Edition, March 2005 (S//NF) |
| DI-1650-83-06 | Chemical, Biological and Radiological Warfare Capstone Threat Assessment, , June 2006 (S//FGI GBR//NF//MR) |
| NASIC-1574-0367-05 | Military Satellite Communications (MILSATCOM) System Threat Assessment Report (STAR), March 2005 (S//NF//MR). |
|  | Defense Intelligence Agency’s (DIA) The Postulated Threat: to U.S. Nuclear Weapon Facilities and Other Selected Strategic Facilities, January 2003 (S//NF//X1) |
|  | Air Force Intelligence Analysis Agency (AFIAA's) Worldwide Asymmetric Threat to USAF Installations, Personnel and Resources, August 2005 (S//NF). |

# Technical Requirements

For the purpose of this specification, the MMP Upgrade terminal EHF/AEHF function will be referred to as “the terminal”.   This document uses shall and will in the following ways: “shall” is a requirement on the terminal; “will” is a desire for a capability or intent to complete an activity with an assignment of responsibility.  All “shall” statements are considered thresholds unless otherwise noted. The terminal is required to meet all requirements in the main body of this TRD and in Section 6, Appendix A of this TRD.

## Terminal Architecture

### System Flexibility

The MMP Upgrade terminal program has a goal to design a terminal with modularity and flexibility to allow ease of upgrade to operation with advanced systems (e.g., Transformational Communications Satellites) with only minimal changes.  This includes the ability to accommodate changes to modulation mode, data rate, network protocol(s), message formats, and terminal definition data, as well as equipment replacement/upgrades.

### Software Communications Architecture (SCA) Requirements

The terminal software shall [1] be fully compliant with the MSRC-5000SCA, v2.2.2

## TERMINAL EXTERNAL INTERFACES

The terminal shall [1] interface with the Higher Authority Communications/Rapid Message Processing Element (HAC/RMPE) station in accordance with D25-92853-V1-C modified as necessary to accommodate higher data rates available with AEHF XDR and the added services provided by the terminal without adversely impacting end-to-end receive message processing time or Probability of Correct Message Receipt (PCMR).

The terminal shall [2] interface to the HAC/RMPE system to automatically forward EAMs, Force Direction Message for Retargeting, Force Management messages, and MMPU-generated  alarm/advisory messages for display to the operator.   Alarm/advisory messages shall [3] also be displayed on the terminal operator control unit without any audible alarm.  The terminal shall [4] interface to HAC/RMPE for transmission of NEREP, other force report-back messages, and FM messages over the specified transmission networks.

In the event of HAC/RMPE primary processor failure, the terminal interface shall [5] be provided to the WSCC backup printer, in accordance with paragraphs 3.1.4.7 and 3.1.4.7.1 of S-133-09259.

The terminal physical and electrical interfaces shall [6] be compatible with all existing Missile Alert Facility (MAF) equipment, including topside equipment, in accordance with S-133-09259, paragraph 3.1.4.

The terminal shall [7] provide an external interface to download terminal log data to a PC laptop or similar device for offline review.

## Modes of operation

The terminal shall [1] support the following modes of operation:

| Mode | Characteristics |
| --- | --- |
| Off | No power |
| Initialization | Power applied Diagnostics run |
| Standby | Terminal operational, but not on a satellite |
| Operational | Terminal operational and on the satellite |
| Maintenance | Terminal is not operational and is undergoing maintenance |

## Terminal functional and Performance requirements

### Backwards Compatibility

The terminal shall [1] meet the requirements specified in the MMP legacy terminal System/Subsystem Specification, S-133-09259, except those that are superseded by requirements in this document.

### Interoperability

The terminal shall [1] be interoperable with the Top Secret receive-only EAM network at protected data rates up to 19.2 kbps.

The terminal shall [2] be interoperable with the Top Secret receive-only Force Direction network at protected data rates up to 19.2 kbps.

The terminal shall [3] be interoperable with the Top Secret bi-directional Force Management network at protected data rates up to 19.2 kbps.

The terminal shall [4] be interoperable with the Secret transmit-only Air Force Reportback network at protected data rates consistent with  SI-3135, Section 3.4.2.6.

#### Message Processing Time

The terminal shall [1] support the overall requirements for receive terminal time to process incoming fixed-formatted messages as defined in CJCSI 6811.01A.

### Probability of Correct Message Receipt

The terminal Probability of Correct Message Reception (PCMR) shall [1] be at least 99 percent. Error correction capability gained at the HAC/RMPE from Phonetic Letter Spell Out (PLSO) to Source format and specified number of EAM repeats will be included.  PCMR  shall [2] be maintained within the adverse weather guidelines planned for in the overall EAP JCS Vol VII (e.g. 5 dB D/L Rain loss and 12 dB Uplink Rain loss).

### Communications Services

The terminal shall [1], as a minimum, be capable of providing the following communications simultaneously.  Simultaneous message transmission requirements do not apply when the AFRB channel is actively transmitting.

- participation in the JCS EAM networks (receive only);

- participation in an Air Force Reportback network (transmit only);

- reception of Milstar EHF and AEHF terminal images using over the air data distribution (OTADD);

- participation in the USSTRATCOM Force Direction network (receive only);

- participation in a bi-directional Force Management network;

- reception of both Communications Security (COMSEC)  and Transmission Security (TRANSEC) keys using over the air re-keying (OTAR) for those constellations supporting OTAR;

- automatic detection of agile beam changes and automatic update of agile beam assignment;

- response to an LDR and XDR Terminal Service Membership query, and reporting back the appropriate status information (e.g. Service ID, Satellite ID, D/L Beam, U/L Beam etc).

- participation in either a Point-to-Point or multi-party voice service (Objective);

### Network Support

The terminal shall [1] be capable of supporting a minimum of three receive network services and a minimum of three transmit services simultaneously. The simultaneous transmission requirement is not applicable during the time a message is being transmitted using the AFRB channel.

### EHF/AEHF Satellite Communications

The terminal shall [1] be interoperable with Milstar in accordance with SI-1135, with AEHF in accordance with SI-3135 Volume 1 and 3, with UFO/E in accordance with PMW-146, and with UFO/EE in accordance with PMW-146.

The terminal shall [2] provide at least the Effective Isotropic Radiated Power (EIRP) listed in Table M-9 of SR-3000, Appendix M, for the LCC minus 0.8 dB and shall [3] provide at least the Gain to Noise Temperature ratio (G/T) listed in Table M-9 for the LCC minus 1.1 dB.

The terminal shall [4] be capable of processing all uplink and downlink communications modes and data rates consistent with the terminal’s EIRP and G/T

The terminal shall [5] support data rates up to and including 19.2 kbps each on three simultaneous receive services on AEHF XDR. The terminal shall [6] support data rates up to and including 19.2 kbps for AEHF XDR on expansion ports provided.  The terminal shall [7] support data rates up to and including 19.2 kbps each on two  simultaneous transmit services on AEHF XDR.  The simultaneous transmit requirement does not apply when the AFRB channel is actively transmitting.

The terminal shall [8] support narrowband voice communications using Linear Predictive Coder (LPC-10e) in accordance with FED-STD-1015 and Mixed Excitation Linear Prediction (MELP) in accordance with MIL-STD-3005. (objective)

The terminal shall [9] provide both Point-to-Point and multi-party voice communications services at 2.4 kbps for both the ANDVT  and MELP voice digitization algorithms as specified in MIL-C-28883 and MIL-STD-3005.  (objective)

### Contingency Operations

The terminal shall [1] support all operational contingency scenarios listed below for AEHF and Milstar constellations:

- Emergency rekey;

- Endurance: The capability to conduct communications and satellite control activities in a scintillated environment throughout a specified period;

- Autonomy: Autonomy is the capability of a satellite to continue to provide communications services in the absence of a control capability;

- Satellite or network failure requiring constellation change.

The MMP Upgrade  shall [2] re-establish connectivity using the current satellite and net data parameters without operator assistance after loss of the satellite signal due to net tear down or other satellite connectivity issues.

### AEHF Operations

The terminal shall [1] provide the control and communications functions specified in SR-3000, Appendix M, Table M-23 , as tailored in Appendix A of this document. Appendix A tailoring marks with a “Y” (for “Yes”) those requirements that apply to the terminal.  The terminal shall [2] provide the control and communications functions specified in SI-3135 Volumes 1 and 3, and SR-3300 for operation with AEHF communications satellites.

#### AEHF Order wire and Transparent Messages

The terminal shall [1] support the AEHF Orderwire transmit and receive functions and transparent message transmissions via the access control channel as defined in SI-3135 Volume 1 and SR-3300, Appendix A.

#### Satellite Transitions

Under operator control and automatically at a predetermined date/time, the terminal shall [1] be able to transition from any satellite in any of the identified constellations to any other satellite in any of the identified constellations.

The terminal shall [2] meet the AEHF requirements for time to switch to a new or modified apportionment on the same or a different satellite in any of the Milstar, AEHF, UFO/E and UFO/EE constellations as defined in SR-3000, Appendix M.  Nominal time to load crypto keys external to the terminal is included in calculating the time required for switching from one satellite to another during normal operations or in support of contingency scenarios.  The crypto keys will be available in each LCC, but not necessarily within the terminal.

### Image and Ephemeris Data Management

The terminal shall [1] provide a terminal image and ephemeris database management system with the following capabilities:

- Support of up to 13 satellites, up to three database versions (current, future and modification) for each;

- Support of engineering staff modification of subset of image parameters;

- Support of manually-initiated immediate switching to a new satellite or future database;

- Support of operator-defined date/time for switch to new satellite or future database;

- Support of over-the-air (OTA) and manual database loads and updates;

- Support of initialization, maintenance, and operational modes;

#### Initialization Parameters and Ephemeris

The terminal shall [1] provide the capability for automatic entry and storage of initialization parameter databases for up to 13 satellites from the Milstar, AEHF, UFO/E and UFO/EE constellations from a data storage device (e.g., CD).  The terminal shall [2] provide for manual entry of date, time, TRANSEC, and COMSEC by the operator in the LCC.  Normal data entry prior to acquisition shall [3] be accomplished via the data storage device.

#### Mission Planning Element (MPE) Data

The terminal shall [1] be capable of accepting and processing terminal image data created by the MPE (replacement for ACMS) in the Common Data Set (CDS) format defined in SI-3145, Volume II for the Milstar and AEHF constellations.  Until MPE is available to support the ICBM mission an interim terminal image generation capability that accommodates all specified constellations (MILSTAR, AEHF, UFO/E, UFO/EE) shall [2] be provided to ensure the MMP upgrade can accomplish its mission with CDS-compliant terminal image data. The ACMS requirement from the MMP legacy System/Subsystem Specification, S-133-09259, does not apply to MMPU.

#### UFO/E/EE Image Data

The terminal shall [1] provide the capability to input, manage, and update image data required to operate with the UFO/E/EE satellite constellations.

#### Over-the-Air Data Distribution (OTADD)

The terminal shall [1] provide the capability for reception, storage, and management of OTADD terminal images.   Following successful acquisition, the terminal shall [2] be capable of automatically requesting and accepting ephemeris via the appropriate access control messages.  The terminal shall [3] automatically save new ephemeris data from either a data storage device or OTA for both currently active and contingency satellites.

Each OTADD event shall [4] be logged automatically.

### Cryptographic Equipment

The terminal shall [1] support all Milstar, AEHF, and UFO/E/EE embedded and external crypto requirements for TRANSEC and COMSEC as identified in SI-1135, SI-3135 Volumes 1 and 3, and PMW 146-S-002.

In support of legacy and planned operations, the terminal shall [2] be interoperable with AEHF satellites and other terminals that utilize the AEHF COMSEC/TRANSEC System (ACTS) chipset in accordance with K00000531, and the KGV-136 or ACTS VPC in accordance with K00000552 for MILSTAR and Advanced EHF COMSEC/TRANSEC functions.

The terminal shall [3] incorporate and, where protection is required, utilize cryptographic equipment approved by National Security Agency (NSA) for terminal operation to provide protection for all information being transmitted from the terminal or received by the terminal.

#### COMSEC/Transmission Security (TRANSEC) Fill Device

The terminal shall [1] be capable of accepting input from existing COMSEC/TRANSEC fill devices.

The terminal shall [2] support loading of COMSEC keys via NSA DS-101 formats using a device (e.g.,  AN/PYQ-10 Simple Key Loader, AN/CYZ-10 Data Transfer Device) and also using media (CD-ROM, etc.) that meet the key specification for the supported network and are provided by a NSA-approved delivery method.

#### Over-The-Air Rekeying (OTAR)

The terminal shall [1] be capable of receiving black COMSEC/TRANSEC keys via OTAR for those constellations supporting OTAR.

#### Key Management

The terminal shall [1] provide Key Management capabilities to support EHF and AEHF crypto period transitions automatically where applicable and to support nuclear contingency scenarios (e.g. crypto keys required to support operation for at least 90 days) to be defined in a terminal Key Management Plan.  The terminal shall [2] manage black and red keys for all identified constellations to include contingency constellations.  The terminal shall [3] automatically save new key data from either a key fill device or OTAR for both currently active and contingency satellites.  The terminal shall [4] be capable of integrating key updates into the currently stored keys as appropriate.

For each satellite image database, a summary of the keys available for each crypto-period shall [5] be maintained.

The crypto Key Management capability shall [6] be available to support all modes of operation (Initialization, Maintenance, Stand-by, and Operational.)

Events that in any way change the keys under terminal management, via OTAR or COMSEC/TRANSEC fill devices, shall [7] be logged automatically.

The crypto key management capability shall [8] support all key-related functions (e.g., yearly endorsement for compatible AEHF end crypto units (ECUs)) required for the identified constellations.

The MMP Update key management capability shall [9] be compliant with AEHF requirements as defined in the AEHF KMP and AEHF KMSD versions in place at the time of the MMP Upgrade contract award.

#### Cryptographic Algorithms for COMSEC and TRANSEC

The MMP Upgrade terminals will be required to be backward compatible to operate with legacy MILSTAR terminals and will be required to support key distribution using THORNTON and GKP format.  Additionally, the MMP Upgrade terminals will be required to support OTAR and Manual Key distribution using the AEHF Generic Fill Format (GFF) Key Packages (GKP) key distribution format.  The terminal shall [1] interface with and support functions for the networks and cryptographic algorithms listed below in Table 3.1.

Table 3.1  MMPU Cryptographic Algorithms Table

| Network | Classification | Mode | Crypto Algorithms |
| --- | --- | --- | --- |
| EAM (Rcv only) | TS | LDR, XDR | SAVILLE, MEDLEY, SHILLELAGH |
| FM (Xmt/Rcv) | TS | LDR, XDR | SAVILLE, MEDLEY, SHILLELAGH |
| FD (Rcv only) | TS | LDR XDR | SAVILLE, MEDLEY, SHILLELAGH |
| AFRB (Xmt only) | S | LDR, XDR | KEESEE, MEDLEY, SHILLELAGH |

### Operator Control

The terminal shall [1] provide an Operator Control Interface (OCI) to support operator control functions including the management of operator inputs/outputs such as cryptovariable loading, terminal image update, database parameter changes, and associated status displays.

The OCI shall [2] allow the ICBM Missile Combat Crew Member (MCCM) the flexibility to change images, services, satellites within the same constellation, supported satellite constellations, and keys (red and black), as directed by the applicable network’s communications planner without the need to dispatch a maintenance team.

The OCI shall [3] give the operator the choice of immediately invoking a terminal image change including a satellite switch or specifying a date and time for a change to take place.

The OCI shall [4] provide the operator with the capability to review the current status of the terminal (including mode of operation), active communications services, satellite, active image, and active TRANSEC and COMSEC keys.

The OCI shall [5] support manual upload or update of both terminal image data and black keys from appropriate electronic media for all identified constellations, including alternate constellations that require such data.

During a data entry procedure, the OCI shall [6] provide the capability to call up a display for the review of all data sets from the terminal image being updated.

Since terminal image data and black keys can be stored for multiple constellations, satellites, and services, the OCI shall [7] provide summary information for each terminal image, highlighting the currently active one.

The OCI shall [8] support the following functions: Mode Selection (Initialization, Maintenance, Stand-by, Operational), Cold Starts, Warm Resets, Diagnostics, and AEHF Orderwire.

The OCI shall [9] maintain a running time log that includes entries for each significant event (e.g., additions, modifications, deletions of image or black key databases whether OTA, from electronic media, or directly by the operator; operator initiated actions; changes in mode of operation.)

The OCI shall [10] provide the capability to download log data to media in a format compatible with PC laptop or similar device for offline review.

The OCI shall [11] allow the operator to control the terminal while strapped into an LCC chair.

The OCI shall [12] provide the capability to zeroize all embedded crypto devices with one command and selectively zeroize individual embedded crypto devices.

The OCI shall [13] provide appropriate sets of capabilities for password controlled access by Maintainer and Engineering/SysAdmin categories of personnel.

### VLF/LF Timing Requirement

The terminal shall [1] provide time to the MMP VLF/LF Terminal (MMP-VT) with accuracy to not degrade the MMP-VT’s capability to decrypt VLF Special Modes messages.

### Prime Power

The terminal shall [1] meet the prime power requirements for the MMP-ET specified in section 3.2.7 of S-133-09259.

### Recovery Timing Requirements

The terminal shall [1] provide the capability to switch between supported satellites, rejoin a network, and load applicable crypto keys, satellite images, system software, and other satellite parameters within 10 minutes without relying on a maintenance dispatch.  The terminal shall [2] provide the capability to switch between supported satellites, rejoin a network, and load applicable crypto keys (red and black), satellite images, system software, and other satellite parameters within 5 minutes (Objective) without relying on a maintenance dispatch.

The terminal shall [3] automatically recover to resume tracking and normal operation without upset within 10 minutes and without loss of system parameters, timing, crypto keys, or ephemeris data throughout periods of nuclear induced environments -- to include High Altitude Electro-Magnetic Pulse (HEMP) -- as defined in Appendix 10 of S-133-09259.   The terminal shall [4] automatically recover to resume tracking and normal operation without upset within 5 minutes and without loss of system parameters, timing, crypto keys, or ephemeris data throughout periods of nuclear induced environments -- to include High Altitude Electro-Magnetic Pulse (HEMP) -- as defined in Appendix 10 of S-133-09259 (Objective).

The terminal shall [5] automatically recover to the Operational state within 10 minutes following restoral of satellite and/or network operations.  The terminal shall [6] automatically recover to the Operational state within 5 minutes following restoral of satellite and/or network operations (Objective).

### HAC/RMPE Requirements

The HAC/RMPE shall [1] be the primary method for composition of all Force Reportback formats and of all Force Management messages for transmission over MILSTAR, AEHF, UFO/E, and UFO/EE.  The HAC/RMPE shall [2] process all messages received on the Force Management network.  Received Force Management messages not recognized as EAMs shall [3] be processed as NAMs.  To maximize EAM, FD, and FM network interoperability across the terminal-HAC/RMPE interface, higher data rates within the constraints of the HAC/RMPE architecture are highly desirable.

Visual alarm indications shall [4] be displayed on the WSCC HAC/RMPE visual display unit (VDU) and audible indications shall [5] be generated and silenced through the WSCC.  HAC/RMPE shall [6] generate an alarm output to the MCC via the WSCC System for fault status which would preclude reception/transmission over the MMP Upgrade (MILSTAR, AEHF, UFO/E, or UFO/EE) satellite systems.

Any modifications to or replacement of existing  HAC/RMPE components accomplished as part of the MMP Upgrade effort shall [7] not degrade the performance and the operational availability of the current HAC/RMPE system.  Any components added to the HAC/RMPE system to support new capabilities as part of the MMP Upgrade effort shall [8] be as reliable as equivalent or similar components in the currently fielded HAC/RMPE system.

## Reliability

### Failure Modes

The terminal, whether operable or inoperable, shall [1] not exhibit any failure mode that would cause failure, damage, or degradation of performance of mission critical functions of the legacy MMP-VT. The terminal, whether operable or inoperable, shall [2] not exhibit any failure mode that would cause failure, damage, or degradation of performance of mission critical functions of the interfacing LCC systems.  The terminal, whether operable or inoperable, shall [3] not cause any safety or environmental hazard within the LCC.  The terminal will meet these requirements under nominal conditions, and when exposed to any LCC nuclear or non-nuclear environment specified in S-133-128C.  The failure of the terminal OCI shall [4] not impair established operational functions of the terminal.

### Mean Time Between Critical Failures (MTBCF)

The terminal, to include the antenna and operator control components, shall [1] have an MTBCF requirement of at least 2750 hours while installed and operating 24 hours per day in the expected environment of the MAFs to include both day to day and wartime environments.

The terminal, to include the antenna and operator control components, shall [2] have an MTBCF of at least 5500 hours while installed and operating 24 hours per day in the expected environment of the MAFs to include both day to day and wartime environments (Objective).

The terminal shall [3] not require dispatch of maintenance personnel or the use of missile combat crews to perform periodic preventive maintenance to meet MTBCF requirements.  Critical failures are those failures of the terminal, including any portions of the HAC/RMPE modified by the MMP upgrade, which result in the inability to receive an EAM, receive a force direction message, transmit a force report-back message, receive a force management message, or transmit a force management message.

### Operational Availability

The terminal shall [1] have a Critical Mission operational availability (Ao) of at least 0.99.  Operational availability is defined as:

Ao= MTBCF/ (MTBCF + Mean Restoral Time (MRT) + Mean Logistics Delay Time (MLDT)).

The MLDT for ICBM communications is 24 hours. MRT is calculated and is a combination of repair of critical mission hardware outages and other critical mission restoral actions, such as reboot, restart, etc.  Ao and MTBCF include incidents and downtime from all sources including, at a minimum,  hardware failures, software errors, operator errors, maintenance errors, no-trouble-found outages (intermittent or cannot duplicate problems).  Only non-relevant failures are excepted (see definition in MIL-HDBK-781, paragraph 3.1.5).  Non-relevant failures are failures that, due to their unique nature, are not expected to be encountered in field service.

## Maintainability

### Fault Detection and Isolation

The terminal design shall [1] contain built-in fault detecting, fault indicating, and fault isolating features to enhance maintenance, and to meet the specified quantitative MTBCF, availability and maintainability requirements. The fault detection and isolation capabilities shall [2] be provided by power-up self-test diagnostics, by On-line tests, and if needed, by operator/maintainer observations and selectable tests.

To support repair fault isolation shall [3] be to two (2) or fewer line replaceable units (LRUs) 98 percent of the time, and to one (1) LRU 95 percent of the time. To support repair, fault isolation shall [4] be to one (1) LRU 99 percent of the time (Objective). These requirements represent Fraction of Faults Isolated (FFI).

To support operations, the Fraction of Faults Detected (FFD) in the mission critical path by automated BIT shall [5] be at least 95 percent. The probability of a false BIT report shall [6] be at most 1 percent.

### Mean Time to Repair (MTTR)

At each site the terminal MTTR shall [1] be less than or equal to 30 minutes, and ninety percent of repairs shall [2] be completable in less than 60 minutes. Active repair time includes detection, diagnosis, repair (remove and replace), alignment, checkout and return to on-line status; administrative and logistics delay times are not included.

### Special Support Equipment

Terminal maintenance shall [1] not require any additional special support equipment.

## Survivability

The terminal shall [1] survive without damage/degradation or loss of system parameters, timing, keys or ephemeris data throughout the nuclear induced environments specified in Appendix A of the ILES ORD, including the nuclear induced High Altitude Electromagnetic Pulse (HEMP).

The terminal shall (2) maintain timing and automatically resume tracking and satellite connection to enable the terminal to automatically resume operation without upset due to a nuclear event within a time period less than or equal to the time needed for the propagation channel to recover from the nuclear perturbations.

The terminal shall [3] not degrade the ability of the MMP-VT to automatically recover from a nuclear event.  MMP-VT recovery is defined as the ability to receive EAM messages to the specified PCMR.

Any HAC/RMPE modifications shall [4] meet the full nuclear Hardness and Survivability requirements of S-133-128C.

### Emergency Operation

Any surviving terminal shall [1] be capable of operating throughout the extended and emergency survivable period for the LCC as specified in S-133-128C, Appendix I.

### Surge Protection

Cabling to/from all terminal topside equipment to the LCC shall [1] provide surge protection devices within the Electrical Surge Arrestor (ESA) vault, providing protection required by S-133-128C.  If new or modified ESAs are required for the terminal, those ESAs will be added within the ESA vault as required.

## TEMPEST

The terminal shall [1] meet the TEMPEST requirements specified in S-133-128C and associated appendices.

## Electromagnetic Interference/Electromagnetic Compatibility (EMI/EMC)

The terminal shall [1] comply with the ICBM EMI/EMC requirements of S-133-128C, paragraph 3.2.2.3.

## Software

Terminal and software design shall [1] be accomplished to ensure 50% processor, memory and other system resource availability under continuous load, calculated on average over a 24 hour period and in consideration of peak utilization.  Peak load shall [2] not exceed 80% of available resources. Peak period is defined as any given time slice of 30 seconds within a 5 minute operational window.

## Human System Integration

The terminal shall [1] be developed in accordance with human engineering design criteria for military systems, personnel, equipment and facilities, using MIL-STD-1472 as a guide.

Terminal design shall [2] permit removal and replacement of terminal end items or LRUs without requiring removal of non-failed LRUs, nor requiring any other facilitating maintenance.

### Manpower and Personnel

The terminal shall [1] be operable and maintainable with the current MMP legacy terminal manning levels and skill level requirements at each location for operations, communications maintenance, security police, and support functions.

### Safety

The terminal shall [1] meet the safety requirements specified in paragraphs 3.7.2, 3.7.2.1 and 3.7.2.2 of S-133-09259.

### Two-Person Weight Restriction

Each of the terminal LRUs shall [1] be within the capacity of two persons to lift, move and handle, as specified in S-133-128C.

## Environmental Conditions

### Operational Environment

The terminal shall [1] be designed for the worst case operating environment of the LCC, as described in S-133-09259.

Equipment such as the antenna/RF components, which operate exposed to the environment shall [2] be capable of operating in environments specified in S-133-09259.

### Non-Operational Environment

The terminal shall [1] be designed for the worst case non-operating environment of its host systems.  Equipment shall [2] withstand non-operational environments specified in S-133-09259.

During shipping, handling, transportation, and storage, the equipment shall [3] withstand extremes in temperatures ranging from -65 to +150 degrees Fahrenheit.

## PHYSICAL requirements

The terminal shall [1] meet the physical characteristics as specified in paragraphs 3.2.7 of S-133-09259.

### Rack Mounting

The terminal components located in the LCC shall [1] have provisions for secure mounting in the existing and/or modified MMP equipment rack, or in the vacant remote control unit space located in the Weapon System Control Console (WSCC).

## Packaging, Handling, and Transportation

### Packaging

Terminal packaging shall [1] ensure that the terminal equipment is capable of meeting the shock and vibration requirements of S-133-128C, Appendix II, paragraphs 20.2.13 and 20.2.14.

### Transportation

Packaged equipment shall [1] be capable of transportation by air and land freight between operational units and depot.

## Required Certifications

### NC2-ESI Certification

The terminal and HAC/RMPE modifications shall [1] meet Nuclear Command and Control Extremely Sensitive Information (NC2-ESE) certification requirements as described in CJCSI 323.01B.

### EAM Certification

The terminal and HAC/RMPE modifications shall [1] meet EAM certification requirements as described in CJCSI5119.01B.

### Nuclear Surety

The terminal and HAC/RMPE modifications shall [1] meet all requirements of the Minuteman ICBM Nuclear Surety Processes, using AFI 63-125 and AFPD 91-1.

### Information Assurance Certification

The terminal shall [1] meet the IA NSA certification requirements as described in the MMP Upgrade UIC and TSRD.

### DIACAP Certification

The terminal shall [1] meet DOD information criteria and be designed, implemented, tested, certified and accredited in accordance with DoDI 8510.bb DoDD 8500.1, and DoDI 8500.2, or the DoD certification and accreditation process in place at time of contract award.

### TEMPEST

The terminal shall [1] meet the TEMPEST requirements specified in S-133-128C and associated appendices.

### Reserved

# Quality Assurance Provisions

## Requirements Verification

Each of the requirements defined in this TRD will be validated by one of the following methods:  inspection (I); analysis (A); demonstration (D); and test (T).  The qualification method(s) to be used for each of the performance requirements will be identified in the Verification Cross Reference Matrix (VCRM) and will be subject to Government approval.

(a)	Inspection.  Verification will be performed by visually examining the item, reviewing descriptive documentation, and comparing the appropriate characteristics with a referenced standard to determine conformance to requirements.

(b)	Analysis.  Verification will be performed by evaluation or simulation using mathematical representations, charts, graphs, circuit diagrams, and data reduction.

(c)	Demonstration.  Verification will be performed by operation, movement or adjustment of the item under a specific condition to perform the desired function without recording quantitative data except for check sheets.

(d)	Test.  Verification will be performed through systematic exercising of the applicable item under all appropriate conditions with instrumentation and test data collection, analysis, and evaluation of quantitative data.

### Subsystem Verification Testing

All verification testing will be performed at the system level.

## Acceptance Testing

Acceptance testing will be defined in the Test Plan and Procedures document.  The requirements stated in the VCRM, Table 4.2, will be verified using the applicable test methods.

Table 4.2 Verification Cross Reference Matrix

| Paragraph | I | A | D | T | Remarks |
| --- | --- | --- | --- | --- | --- |
| TBD |  |  |  |  |  |

# List of Acronyms and Abbreviations

| Acronyms and Abbreviations |  |
| --- | --- |
| ACTS | AEHF COMSEC/TRANSEC System |
| AEHF | Advanced Extremely High Frequency |
| AIS | Automated Information Systems |
| API | Application programming Interface |
| ATO | Authority to Operate |
| ATOW | Acquisition and Tracking Orderwire |
| BC | Backward Compatible |
| BER | Bit Error Rate |
| CCA | Circuit Card Assembly |
| CDD | Capability Development Document |
| CDR USSTRATCOM | Commander, United States Strategic Command |
| CDS | Common Data Set |
| CER | Channel Error Rate |
| COMSEC | Communications Security |
| DAA | Designated Approval Authority |
| DIACAP | DoD Information Assurance Certification and Accreditation Process |
| DITSCAP | DOD Information Technology Security Certification and Accreditation Process |
| DoD | Department of Defense |
| EAL | Evaluated Assurance Level |
| EAM | Emergency Action Messages |
| EAP | Emergency Action Procedures |
| EHF | Extremely High Frequency |
| EIRP | Effective Isotropic Radiated Power |
| EMI/EMC | Electromagnetic Interference/Electromagnetic Compatibility |
| ESA | Electrical Surge Arrestor |
| FCA | Functional Configuration Audit |
| FFD | Fraction of Faults Detected |
| FFI | Fraction of Faults Isolated |
| FOUO | For Official Use Only |
| FSDPSK | Filtered Symmetric Differential Phase Shift Keying |
| FSK | Frequency Shift Keying |
| GFF | Generic Fill Format |
| GKP | GFF Key Packages |
| GMSK | Gaussian Minimum Shift Keying |
| GOE | Ground Operating Equipment |
| G/T | Gain to Noise Temperature |
| HAC/RMPE | Higher Authority Communications/Rapid Message Processing Element |
| HEMP | High Altitude Electromagnetic Pulse |
| HGEC | High Gain Earth Coverage |
| HHR | High Hop Rate |
| HRCA | High Resolution Coverage Area |
| IA | Information Assurance |
| ICBM | Intercontinental Ballistic Missile |
| ICD | Interface Control Document |
| ILES | ICBM Launch Control Center EHF System |
| IRS | Interface Requirements Specification |
| JCS | Joint Chiefs of Staff |
| JS | Joint Staff |
| JTA | Joint Technical Architecture |
| JTSS | Joint Terminal System Specification |
| KP | Key Processor |
| LCC | Launch Control Center |
| LDR | Low Data Rate |
| LHR | Low Hop Rate |
| LMD | Local Management device |
| LRU | Line Replaceable Unit |
| MAF | Missile Alert Facility |
| MAJCOM | Major Command |
| MCCM | Missile Combat Crew Member |
| MCS | Mission Control Segment |
| MEECN | Minimum Essential Emergency Communications Network |
| MELP | Mixed Excitation Linear Prediction |
| MER | Message Error Rate |
| MLDT | Mean Logistics Delay Time |
| MMP | Minuteman MEECN Program |
| MMP-ET | MMP EHF Terminal (note: part of MMP) |
| MMP-VT | MMP VLF/LF Terminal (note: part of MMP) |
| MPE | Mission Planning Element |
| MRCA | Medium Resolution Coverage Area |
| MRT | Mean Restoral Time |
| MTBCF | Mean Time Between Critical Failure |
| MTTR | Mean Time to Repair |
| NEREP | Nuclear Execution Reporting |
| NSA | National Security Agency |
| OCI | Operator Control Interface |
| ORD | Operational Requirements Document |
| OTA | Over the Air |
| OTADD | Over the Air Data Distribution |
| OTAR | Over the Air Re-keying |
| PCA | Physical Configuration Audit |
| PCMR | Probability of Correct Message Receipt |
| PGMSK | Pulsed Gaussian Minimum Shift Keying |
| PLSO | Phonetic Letter Spell out |
| RIP | Received Isotropic Power |
| SCA | Software Communications Architecture |
| SHGEC | Super High Gain Earth Coverage |
| SIOP | Single Integrated Operational Plan |
| SPO | System Program Office |
| STAR | System Threat Assessment Report |
| TDMA | Time Division Multiple Access |
| TOD | Time of Day |
| TPCD | Terminal Planning Constraints Document |
| TRANSEC | Transmission Security |
| TRD | Technical Requirements Document |
| TT&C | Telemetry, Tracking and Command |
| UFO | UHF Follow-On |
| USSTRATCOM | United States Strategic Command |
| VPC | Variable Performance Chipset |
| WSCC | Weapon System Control Console |
| XDR | Extended Data Rate |

# APPENDIX A

# Terminal Requirements from SR-3000, Appendix M, Table M-23, Tailored for the MMP Upgrade Terminal

All references to tables and appendices in the ensuing text unless specifically noted refer to SR-3000, its tables and appendices.

| No. | REQID | Requirement | MMP | Comments |
| --- | --- | --- | --- | --- |
| 1 | TRA01025 | Secret | Y |  |
| 2 | TRA01030 | LDR terminals shall [1] be capable of receiving at both HHR and LHR. | Y |  |
| 3 | TRA01065 | LDR terminals shall [1] support C0 data rates from 75 to 2400 bps and C1 data rates from 75 to 300 bps. | Y |  |
| 4 | TRA01070 | LDR Terminals shall [1] interface with the space segment in accordance with the Payload to Terminal ICD, Volume 3. While not all features will be implemented by all terminals, terminals will implement features in accordance with the ICD. The required core set of features necessary for interoperability is defined in SR-2300. | Y |  |
| 5 | TRA01075 | LDR terminals shall [1] be capable of receiving data at any downlink mode that is more robust than the least robust mode they can receive using downlink modes described in ICD-3. | Y |  |
| 6 | TRA01080 | Not applicable for MMP Upgrade | N |  |
| 7 | TRA01450 | Those terminals which are logged on and are actively participating in Strategic communications services, as defined in Appendix D, shall [1] be capable of sustaining those communications when the satellite is repositioning at a maximum of 6 degrees per day. | Y |  |
| 8 | TRA01470 | Not applicable for MMP Upgrade | N |  |
| 9 | TRA01490 | Not applicable for MMP Upgrade | N |  |
| 10 | TRA01660 | Not applicable for MMP Upgrade | N |  |
| 11 | TRA01670 | Not applicable for MMP Upgrade | N |  |
| 12 | TRA01770 | Not applicable for MMP Upgrade | N |  |
| 13 | TRA01860 | AEHF terminals shall [1] support data rates selected from the following data rates expressed in bps: 75, 150, 300, 600, 1.2k, 2.4k, 4.8k, 9.6k, 16k, 19.2k, 32k, 64k, 128k, 256k, 512k, 1024k, 1544k, 2048k, 4096k, and 8192k, up to the maximum capability of the terminal. | Y | MMP upgrade shall [2] support the processing of all LDR, BC LDR, and XDR waveforms and data rates. |
| 14 | TRA01865 | Not applicable for MMP Upgrade | N |  |
| 15 | TRA01870 | All terminals shall [1] have approved DD1494 to operate the system. | Y |  |
| 16 | TRA01910 | Secret | Y |  |
| 17 | TRA01920 | Not applicable for MMP Upgrade | N |  |
| 18 | TRA01940 | The AEHF terminal shall [1] provide BER of 1E-05 given the following conditions: a) The downlink signal degraded RIP and the corresponding payload uplink Channel Error Rate (CER) shown in Table M-11. b) The downlink modulation mode shown in Table M-11a and the downlink data rate shown in Table M-11b for each terminal type and applicable coverage area. c) A channel interleaver is used which provides a minimum diversity of 6 downlink hops when using the modulation mode shown in Table M-11a for data rates greater than or equal to 32 kbps. Diversity is defined to be the number of distinct downlink high hop rate (HHR) hops over which the chips in a single turbo encoder output block are distributed after channel interleaving. Degraded RIP is defined to be the actual RIP available to the terminal, including the effects of satellite pointing errors to anywhere within the coverage area, reduced to account for payload implementation losses. The BER requirement shall [2] be met assuming an undistorted downlink waveform as defined in ICD-1, given the downlink RF characteristics specified in ICD-1, including payload timing jitter and frequency error, and accounting for uncompensated Doppler frequency error due to satellite orbital motion. | Y |  |
| 19 | TRA01943 | Not applicable for MMP Upgrade | N |  |
| 20 | TRA01947 | Not applicable for MMP Upgrade | N |  |
| 21 | TRA01948 | Not applicable for MMP Upgrade | N |  |
| 22 | TRA01950 | Terminals shall [1] support the range of terminal identification numbers defined in ICD-1. | Y |  |
| 23 | TRA01990 | An XDR terminal shall [1] be capable of transmitting GMSK waveforms at the appropriate data rates given in ICD-1. | Y |  |
| 24 | TRA02000 | An XDR terminal shall [1] be capable of transmitting PGMSK, FSDPSK and FSK waveforms in accordance with ICD-1. | Y |  |
| 25 | TRA02010 | The spectral purity, spurious signals, out-of-channel and out-of-band emissions shall [1] be in accordance with ICD-1. | Y |  |
| 26 | TRA02050 | XDR terminals shall [1] be capable of transmitting communications data, time probes, and access control messages using the uplink frame structure specified in ICD-1. | Y |  |
| 27 | TRA02060 | XDR terminals shall [1] be capable of transmitting into any XDR uplink access and routing data from any XDR input port to any uplink XC0 access providing no conflict exists with probing or access control. | Y |  |
| 28 | TRA02090 | Terminals shall [1] support signal randomization for uplink frequency hopping and time permutations as described in ICD-1. | Y |  |
| 29 | TRA02110 | Terminals shall [1] encrypt uplink access control messages as described in ICD-1. | Y |  |
| 30 | TRA02115 | Terminals shall [1] support transmission on uplink channels that use the TDMA frame formats defined in ICD-1 provided that each terminal is required to transmit on at most a single channel during a hop. | Y |  |
| 31 | TRA02130 | XDR terminals shall [1] be capable of receiving synchronization hops associated with their logon beam on any XDR downlink access of a single downlink chain and communications data including XC3s and ATOWs on any XDR downlink access on their logon beam in that same chain. A chain is defined as a modulator, transmitter and antenna. | Y |  |
| 32 | TRA02140 | XDR terminals shall [1] be capable of receiving data at any downlink mode that is equal to or more robust than the least robust mode they can receive using downlink modes described in ICD-1. | Y |  |
| 33 | TRA02150 | XDR terminals shall [1] be capable of receiving downlink access control (XC3) and ATOW messages at any downlink mode that is equal to or more robust than the least robust mode they can receive using downlink modes described in ICD-1. | Y |  |
| 34 | TRA02160 | Terminals shall [1] support signal randomization for downlink frequency hopping and time permutations as described in ICD-1. | Y |  |
| 35 | TRA02170 | Terminals shall [1] decover AEHF ATOW messages as described in ICD-1. | Y |  |
| 36 | TRA02180 | Terminals shall [1] decover downlink access control messages as described in ICD-1. | Y |  |
| 37 | TRA02190 | Terminals shall [1] decrypt downlink access control messages as described in ICD-1. | Y |  |
| 38 | TRA02192 | Terminals shall [1] support BC COMSEC/TRANSEC functions in accordance with ICD-2, ICD-3, and KGV-136 Ground Operating Equipment (GOE) or Variable Performance Chipset (VCP) ICD. | Y | Except ICD-2 (MDR) |
| 39 | TRA02194 | Terminals shall [1] support the use of BC COMSEC/TRANSEC algorithms for both AEHF and BC signal transmission randomization and BC access control and ATOW message protection simultaneous with AEHF KGV-136 GOE or VPC algorithms for AEHF ATOW cover, access control message ID and CRC cover, and XC2/XC3 message bit field encryption, as described in ICD-1, ICD-2, and ICD-3. | Y | ICD-1 and ICD-3 only |
| 40 | TRA02250 | XDR terminals shall [1] be capable of acquiring and tracking the downlink hop and symbol timing, as described in ICD-1, in the intended environment (weather, jamming, scintillation) of operations. | Y |  |
| 41 | TRA02255 | During time tracking, terminals shall [1] be able to maintain communications and synchronization without reacquiring, logging off or tearing down services for an ATOW outage not to exceed the number of consecutive messages indicated in Table M-32. Synchronization hops will be maintained. | Y |  |
| 42 | TRA02260 | The XDR carrier frequencies and waveform timing are satellite referenced. XDR terminals shall [1] transmit XDR uplink signals with time and frequency accuracy in accordance with ICD-1. | Y |  |
| 43 | TRA02280 | Not applicable for MMP Upgrade | N |  |
| 44 | TRA02290 | Not applicable for MMP Upgrade | N |  |
| 45 | TRA02295 | Not applicable for MMP Upgrade | N |  |
| 46 | TRA02300 | Not applicable for MMP Upgrade | N |  |
| 47 | TRA02301 | Not applicable for MMP Upgrade | N |  |
| 48 | TRA02302 | Not applicable for MMP Upgrade | N |  |
| 49 | TRA02304 | After the downlink is acquired, the terminals shall be capable of acquiring the uplink and logging on to the system within t times shown in Table M-28. This is dependent on the payload performance and interface being specified in accordance with the system specification and ICD-1. A: Coverage types = full time and beam share MRCA, and HRCA. Assumes appropriate keys are loaded and probe type 0 or 1 for WB PSK channels. | Y |  |
| 50 | TRA02310 | Secret | Y |  |
| 51 | TRA02330 | XDR terminals shall [1] be capable of performing XDR uplink acquisition and time tracking to synchronize and maintain XDR uplink timing and frequency with the satellite as defined in ICD-1 in the intended weather, jamming and scintillation. | Y |  |
| 52 | TRA02335 | At the start of uplink acquisition, XDR terminals shall [1] have a maximum terminal rate error of +/-5E-8. | Y |  |
| 53 | TRA02360 | Initial uplink acquisition signal transmission shall [1] satisfy IDT0250 as defined in ICD-1 under unstressed and stressed (jamming, scintillation) environments. The unstressed and jamming environments are specified in Section 3. The scintillation environment is specified in Appendix B-3.1.3. | Y |  |
| 54 | TRA02380 | In the unstressed environment Terminals shall [1] be capable of receiving downlink messages at the selected XC3/ATOW mode with a message error rate (MER) of no greater than 0.01 when the messages are transmitted in the modes allowed in Table M-29 given the downlink signal RIP Shown in Table M-11 and RIP adjustments as follows: 4+392 MODE -1.4dB DPA and -0.8dB GDA. The unstressed environment is specified in Section 3. | Y |  |
| 55 | TRA02381 | In the scintillation environment Terminals shall [1] be capable of receiving downlink messages at the selected XC3/ATOW mode with a message error rte (MER) of no greater than 0.01 when the messages are transmitted in the modes allowed in Table M-30 given the downlink signal RIP shown in Table M-11 under the scintillation environment. The scintillation environment is specified in Appendix B. | Y |  |
| 56 | TRA02382 | Not applicable to MMP Upgrade | N |  |
| 57 | TRA02383 | An XDR terminal shall [1] have a maximum terminal rate error when tracking as specified in Table M-33 to support probe types 0 through 6. | Y |  |
| 58 | TRA02385 | XDR terminals shall [1] be capable of accessing the System via the HGEC. Access for AEHF terminals includes interfacing with the satellite for the purpose of requesting SHGEC, full-time steerable HGEC, MRCA, or HRCA coverage. | Y |  |
| 59 | TRA02387 | Secret | Y |  |
| 60 | TRA02388 | Secret | Y |  |
| 61 | TRA02400 | Terminals shall [1] be able to be simultaneously logged on to the BC LDR and/or BC MDR System, as well as the AEHF System. | Y | MMP upgrade will not support MDR services. |
| 62 | TRA02405 | Not applicable to MMP Upgrade | N |  |
| 63 | TRA02407 | Terminals simultaneously logged on to the BC LDR and/or BC MDR System, as well as the AEHF System that support access control encapsulation, shall [1] select whether to send XC2s, LDR probes, MDR probes or XDR probes if the fields overlap in the uplink frame and in time | Y | MMP upgrade will not support MDR services. |
| 64 | TRA02410 | Selected terminals shall [1] be able to participate in BC LDR, BC MDR, and XDR services, while processing a single downlink channel, as described in ICD-1. | Y | MMP upgrade will not support MDR services. |
| 65 | TRA02420 | XDR terminals shall [1] be capable of communicating at an elevation of 5 degrees or higher with satellites in orbits that are geosynchronous with any right ascension of ascending node, any satellite inclination up to 5 degrees, and any eccentricity from 0.0 to 0.013. This excludes outages due to satellite handover. Below 20 degrees elevation, degraded performance allowed. | Y |  |
| 66 | TRA02425 | Terminals shall [1] be capable of activating and participating in LDR, MDR and/or XDR services that span multiple satellites and beam types. | Y | MMP upgrade will not support MDR services. |
| 67 | TRA02427 | Terminals shall [1] be capable of operating in a constellation of as many as 10 satellites, including as many as two Milstar 1 (LDR only) satellites, as many as three Milstar II (LDR and MDR) satellites and as many as five AEHF satellites. | Y | MMP upgrade shall [2] be capable of operating on the 5 Milstar and 5 AEHF satellites plus 3 UFO/E/EE satellites. |
| 68 | TRA02450 | XDR terminals shall [1] be capable of requesting, receiving, and storing ephemeris for any Milstar or AEHF satellite using access control messages as specified in the Advanced EHF Payload-to-Terminal Interface Control Document ICD-1. | Y |  |
| 69 | TRA02500 | XDR terminals with the appropriate priority level shall [1] be able to assume the role of Antenna Controller (AC) to control the pointing of a steerable antenna beam, as defined in ICD-1. | N |  |
| 70 | TRA02504 | XDR AC terminals pointing steerable XDR antenna beams as defined in ICD-1 shall [1] always set the “Send Antenna pointing Report Coordinates” option to “No” unless specifically requested by the terminal operator to set the “Send Antenna pointing Report Coordinates” to “Yes”. | N |  |
| 71 | TRA02510 | Not applicable to MMP Upgrade | N |  |
| 72 | TRA02640 | LDR terminals shall [1] process and respond to LDR Terminal ID to Service Membership (TSVC) TM-C3 requests using the TSVC TM-C2 as specified in the Advanced EHF Payload-to-Terminal LDR Backwards Compatible Interface Control Document ICD-3. Terminals may provide a capability to inhibit the response to the LDR terminal ID to Service Membership (TSVC) TM-C3 requests. | Y |  |
| 73 | TRA02700 | The Terminal Segment shall [1] prevent terminal operators from modifying the maximum XDR priority provided in the terminal image. | Y |  |
| 74 | TRA02702 | Selected XDR terminals shall [1] support uplink SNR measurement, as defined in ICD-1. | Y |  |
| 75 | TRA02704 | XDR terminals shall [1] be capable of supporting transparent messages as defined in ICD-1. | Y |  |
| 76 | TRA02706 | XDR terminals shall [1] be capable of using the full-time steerable HGEC or SHGEC for communication service per ICD-1. | Y |  |
| 77 | TRA02740 | Not applicable to MMP Upgrade | N |  |
| 78 | TRA02742 | Not applicable to MMP Upgrade | N |  |
| 79 | TRA02745 | XDR Terminals designated as fence controller shall [1] be able to use the protocols to create one or more sub-fences… | N |  |
| 80 | TRA02750 | Not applicable to MMP Upgrade | N |  |
| 81 | TRA02760 | Not applicable to MMP Upgrade | N |  |
| 82 | TRA02761 | Not applicable to MMP Upgrade | N |  |
| 83 | TRA02762 | Selected XDR terminals shall [1] be capable of supporting communication services which employ any combination of unfenced resources and fenced resources from fences, sub-fences, and sub-sub-fences, as defined in ICD-1. | Y |  |
| 84 | TRA02763 | Not applicable to MMP Upgrade | N |  |
| 85 | TRA02764 | Not applicable to MMP Upgrade | N |  |
| 86 | TRA02765 | Not applicable to MMP Upgrade | N |  |
| 87 | TRA02766 | Not applicable to MMP Upgrade | N |  |
| 88 | TRA02767 | Not applicable to MMP Upgrade | N |  |
| 89 | TRA02768 | Talker terminals participating in flooded XDR services identified in Appendix D as Constellation Management, Force Management, Decision Making, Situation Monitoring, Force Direction, or Planning, when notified by an SRN-XC3 that the topology of the constellation has changed from a ring to string due to a failure, shall [1] wait at least 25 seconds but no longer than 60 seconds before preparing a reconfiguration request. The wait time excludes XC2 cycle and transmission times. | Y |  |
| 90 | TRA02770 | Not applicable to MMP Upgrade | N |  |
| 91 | TRA02771 | All terminals, except the Navy terminals, shall [1] be able to create and make ready for transmission within 3 seconds an XC2 message, to be transmitted in the next available slot. The time starts when the terminal receives a command from terminal operator, after the completion of MPE command processing per TRA02784, after the completion of XC3 processing as specified in TRA02775 or TRA-2776, or after completing generation and transmission of an XC2 when a particular command requires multiple TM XC2s to be transmitted. | Y |  |
| 92 | TRA02775 | The terminal shall [1] be able to process an XC3 within 3 seconds of reception. | Y |  |
| 93 | TRA02776 | Not applicable to MMP Upgrade | N |  |
| 94 | TRA02778 | Not applicable to MMP Upgrade | N |  |
| 95 | TRA02782 | Not applicable to MMP Upgrade | N |  |
| 96 | TRA02784 | Not applicable to MMP Upgrade | N |  |
| 97 | TRA02786 | Not applicable to MMP Upgrade | N |  |
| 98 | TRA02787 | Not applicable to MMP Upgrade | N |  |
| 99 | TRA02788 | Not applicable to MMP Upgrade | N |  |
| 100 | TRA02792 | Terminals shall [1] be capable of starting the orderwire protocol to generate an unclassified free text message of up to and including 200 characters in length within 1 minute of the operator executing the send command. The orderwire protocol is described in the JTSS. | Y |  |
| 101 | TRA02793 | Terminals shall [1] be capable of displaying an unclassified free text orderwire message of up to and including 200 characters in length within 1 minute of receiving the last orderwire XC3. The orderwire protocol is described in the JTSS. | Y |  |
| 102 | TRA02794 | XDR terminals shall [1] support rapid reconfiguration of HDX services (RROW) as defined in ICD-1. | Y |  |
| 103 | TRA02796 | Selected XDR terminals shall [1] support downlink bundling as defined in ICD-1. | Y |  |
| 104 | TRA02800 | Not applicable to MMP Upgrade | N |  |
| 105 | TRA02805 | Not applicable to MMP Upgrade | N |  |
| 106 | TRA02810 | Not applicable to MMP Upgrade | N |  |
| 107 | TRA02812 | Not applicable to MMP Upgrade | N |  |
| 108 | TRA02813 | Not applicable to MMP Upgrade | N |  |
| 109 | TRA02814 | Not applicable to MMP Upgrade | N |  |
| 110 | TRA02815 | XDR terminals shall [1] have the capability to request AEHF COMSEC/TRANSEC key updates and changes, as defined in ICD-1. | Y |  |
| 111 | TRA02820 | Not applicable to MMP Upgrade | N |  |
| 112 | TRA02890 | Secret | Y |  |
| 113 | TRA02900 | FOUO | Y | HEMP-related performance. |
| 114 | TRA02930 | Those terminals required to transmit on multiple services defined in Appendix D shall [1] have the capability to transmit on four services simultaneously at a minimum. | Y |  |
| 115 | TRA02932 | Not applicable for MMP Upgrade | N |  |
| 116 | TRA02950 | XDR Terminals shall [1] interface with the space segment in accordance with the Payload to Terminal ICD, Volume 1. While not all features will be implemented by all terminals, terminals will implement features in accordance with the ICD. The required core set of features necessary for interoperability is defined in the JTSS. | Y |  |
| 117 | TRA02955 | Terminal shall [1] support the use of 25% Time Shared MRCAs. | Y |  |
| 118 | TRA02970 | Not applicable to MMP Upgrade | N |  |
| 119 | TRA02990 | The terminal segment shall [1] be able to receive image data in a format that is common to all AEHF-compatible terminals as specified in MCS to Terminal Segment ICD, SI-3245 Vol II. | Y |  |
| 120 | TRA02992 | Not applicable to MMP Upgrade | N |  |
| 121 | TRA02994 | The terminal shall [1] accept and process terminal image data received over AEHF XC0 communications resources as defined in the MCS to Terminal Segment ICD, SI-3145 Volume II and the Joint Terminal Segment Specification (JTSS). | Y |  |
| 122 | TRA02996 | The terminal shall [1] be capable of requesting the distribution of terminal image data over AEHF XC0 communications resources, as defined in the MCS to Terminal Segment ICD, SI-3145 Volume II, the AEHF Payload to Terminal ICD, ICD-1 and the Joint Terminal Segment Specification (JTSS). | Y |  |
| 123 | TRA03000 | Not applicable to MMP Upgrade | N |  |
| 124 | TRA03045 | Not applicable to MMP Upgrade | N |  |
| 125 | TRA03050 | Not applicable to MMP Upgrade | N |  |
| 126 | TRA03060 | The Terminal Segment shall [1] be capable of accepting and processing an over the air terminal data distribution within 10 minutes of receipt of the terminal data. The time period starts when the terminal has received the terminal data and ends when the terminal is ready to use the received terminal data. | Y |  |
| 127 | TRA03110 | All user terminals shall [1] be capable of interoperating as defined in the JTSS at all non-user-defined data rates up to the maximum capability of the terminal. | Y |  |
| 128 | TRA03150 | The terminals shall [1] meet the interoperable terminal requirements detailed in the Joint Terminal Segment Specification (JTSS). | Y | The terminal shall [2] meet the interoperability requirements in SR-2300. |
| 129 | TRA03180 | Not applicable to MMP Upgrade | N |  |
| 130 | TRA03190 | Not applicable to MMP Upgrade | N |  |
| 131 | TRA03200 | Not applicable to MMP Upgrade | N |  |
| 132 | TRA03235 | Not applicable to MMP Upgrade | N |  |
| 133 | TRA03240 | In the HGEC Air Force Bomber/LCC Terminals shall [1] be capable of transmitting Bomber reportback messages as defined in ICD-1 and ICD-3. | Y | AFRB requirement. |
| 134 | TRA03250 | Not applicable to MMP Upgrade | N |  |
| 135 | TRA03260 | Not applicable to MMP Upgrade | N |  |
| 136 | TRA03290 | Not applicable to MMP Upgrade | N |  |
| 137 | TRA03292 | Not applicable to MMP Upgrade | N |  |
| 138 | TRA03293 | Not applicable to MMP Upgrade | N |  |
| 139 | TRA03294 | Not applicable to MMP Upgrade | N |  |
| 140 | TRA03340 | The AEHF terminal shall [1] provide BER of 1E-5 (or 1E-8 for CER=0.03 for WIN-T SMART-T in HRCA and MRCA) given the downlink signal RIP and the uplink Channel Error Rate as shown in Table M-17 when it is operating at the specified user data rate in System Uplink AJ Requirements, as in Table M18b, for coverage areas and the downlink modes shown in Table M-18a. The uplink channel errors are due to uplink jamming environment as are defined in the System AJ Requirements. The downlink RF characteristics are defined in ICD-1. | Y | Reference to classified table. Verify that Tables M-17, M-18a and M-18b entries for LCC environment are appropriate and do not have adverse impact in other areas such as reuse of existing antenna suite and ancillary equipment. |
| 141 | TRA03380 | Not applicable to MMP Upgrade | N |  |
| 142 | TRA03382 | The AEHF terminals that appear in Appendix D data nets shall [1] each provide BER of 1E-5 using downlink modes given uplink and downlink conditions as shown in Table M-26. Uplink channel errors, when applicable, may be due to uplink fading (independent of downlink fade) environment. The fading environment is defined in Appendix B-8.3.1.3. The downlink RF characteristics are defined in ICD-1. | Y | Nuclear effects requirement; we need verification of entries for LCC environment. Note that other AF strategic terminals include this requirement. |
| 143 | TRA03384 | The terminals that appear in Appendix D voice nets when using the interleaver as described in TRA03410 shall [1] … | N |  |
| 144 | TRA03390 | FOUO | Y |  |
| 145 | TRA03395 | Not applicable to MMP Upgrade | N |  |
| 146 | TRA03397 | Not applicable to MMP Upgrade | N |  |
| 147 | TRA03410 | Not applicable to MMP Upgrade | N |  |
| 148 | TRA03415 | Not applicable to MMP Upgrade | N |  |
| 149 | TRA03430 | Not applicable to MMP Upgrade | N |  |
| 150 | TRA03440 | Selected authorized XDR terminals shall [1] be capable of generating unclassified free text messages of up to and including 200 characters in length and specifying the intended logged on destination XDR terminal(s) as defined in ICD-1. | Y |  |
| 151 | TRA03560 | All XDR terminals shall [1] support interoperable communications with those terminals that comply with ICD-1, JTSS, and the MCS to Terminal Segment ICD, SI-3145. Terminal unique information is defined in the Terminal Planning Constraints Document (TPCD). | Y |  |
| 152 | TRA03590 | Not applicable to MMP Upgrade | N |  |
| 153 | TRA03592 | Not applicable to MMP Upgrade | N |  |
| 154 | TRA03620 | Not applicable to MMP Upgrade | N |  |
| 155 | TRA03625 | Not applicable to MMP Upgrade | N |  |
| 156 | TRA03650 | Not applicable to MMP Upgrade | N |  |
| 157 | TRA03660 | Not applicable to MMP Upgrade | N |  |
| 158 | TRA03670 | Not applicable to MMP Upgrade | N |  |
| 159 | TRA03675 | Not applicable to MMP Upgrade | N |  |
| 160 | TRA03700 | Not applicable to MMP Upgrade | N |  |
| 161 | TRA03710 | Not applicable to MMP Upgrade | N |  |
| 162 | TRA03730 | Not applicable to MMP Upgrade | N |  |
| 163 | TRA03740 | Not applicable to MMP Upgrade | N |  |
| 164 | TRA03750 | Not applicable to MMP Upgrade | N |  |
| 165 | TRA03760 | Not applicable to MMP Upgrade | N |  |
| 166 | TRA03770 | Not applicable to MMP Upgrade | N |  |
| 167 | TRA03772 | Not applicable to MMP Upgrade | N |  |
| 168 | TRA03774 | Not applicable to MMP Upgrade | N |  |
| 169 | TRA03790 | Not applicable to MMP Upgrade | N |  |
| 170 | TRA03800 | Not applicable to MMP Upgrade | N |  |
| 171 | TRA03802 | Not applicable to MMP Upgrade | N |  |
| 172 | TRA03820 | Not applicable to MMP Upgrade | N |  |
| 173 | TRA03822 | The Terminal Segment shall [1] ensure that terminals receiving TOP SECRET baseband keys provide security domain separation between TOP SECRET baseband processing and access control messages. | Y |  |
| 174 | TRA03823 | The Terminal Segment shall [1] prevent terminal operators from modifying the terminal ID provided in the terminal image. | Y |  |
| 175 | TRA03824 | The Terminal Segment shall [1] prevent terminal operators from modifying the maximum XDR precedence provided in the terminal image. | Y |  |
| 176 | TRA03825 | Not applicable to MMP Upgrade | N |  |
| 177 | TRA03826 | Not applicable to MMP Upgrade | N |  |
| 178 | TRA03830 | Terminals transmitting or receiving BC message traffic TOD encrypted with the KEESEE/KGV-136/VPC shall [1] do so in accordance with ICD-3. | Y |  |
| 179 | TRA03835 | XDR terminals shall [1] be capable of acquiring in the acquisition HGEC followed by logging onto and joining service on any other communications beam that is operating on either uplink TRANSEC key. | Y |  |
| 180 | TRA03836 | XDR terminals shall [1] be capable of acquiring and communicating in any beam that supports acquisition, access controls, and communication using the one and only one uplink TRANSEC key that is currently configured to that beam. | Y |  |
| 181 | TRA03838 | Terminals logged onto the acquisition HGEC shall [1] be able to execute the antenna control and antenna pointing protocols as described in the Payload-to-Terminal ICD-1. | Y |  |
| 182 | TRA03840 | XDR Terminals shall [1] be capable of operating on separate uplink and downlink TRANSEC keys. | Y |  |
| 183 | TRA03842 | Not applicable to MMP Upgrade | N |  |
| 184 | TRA03844 | Terminals shall [1] be able to simultaneously store and use multiple group session keys. | Y |  |
| 185 | TRA03852 | Each active AEHF terminal shall [1] support OTAR success by implementing the OTAR protocols of ICD-1 and complying with the OTAR constraints of ICD-1. | Y |  |
| 186 | TRA03854 | Each active AEHF terminal shall [1] monitor the status of its cryptographic keys. | Y |  |
| 187 | TRA03856 | Secret | Y |  |
| 188 | TRA03895 | Terminals shall [1] support Benign fill of keys from the LMD/KP. | Y |  |
| 189 | TRA03900 | The terminal, described in its Security Target, shall be able to become evaluated at Evaluation Assurance Level (EAL) 3 or greater in accordance with ISO/IEC 15408. | N |  |
| 190 | TRA03903 | The terminal shall [1] retain keys distributed to the terminal via OTAR for the duration of the key effectivity, or until zeroized or erased by the operator. | Y |  |
| 191 | TRA03905 | Terminals that employ GPS shall [1] comply with CJCSI-6140.01 and CJCSI-6130.01 requirements. | N* | *Only applies if MMP Upgrade design incorporates GPS as timing source (not anticipated). |
| 192 | TRA03907 | Not applicable to MMP Upgrade | N |  |
| 193 | TRA03910 | Not applicable to MMP Upgrade | N |  |
| 194 | TRA03925 | Not applicable to MMP Upgrade | N |  |
| 195 | TRA03940 | Not applicable to MMP Upgrade | N |  |
| 196 | TRA03960 | Terminals that interface with the KGV-136 GOE (Ground Operational Equipment) shall [1] do so in accordance with the KGV-136 GOE ICD. | TBD | Will depend upon MMP Upgrade architecture. |
| 197 | TRA03970 | Terminals that interface with the Variable Performance Chipset shall [1] do so in accordance with the Variable Performance Chipset ICD. | TBD | Will depend upon MMP Upgrade architecture. |