Appendix C

# Characterization of Customer Community Networks

- Table C-1.  Public/Commercial Networks (Satellites)

- Table C-2.  Public/Commercial Networks

- Table C-3.  DoD Networks

- Table C-4.  Networking Technologies

Table C-1.  Public/Commercial Networks (Satellites)

| Acronym | Full Name | Used By | Purpose | User Bandwidth | Multiple Access Methods |
| --- | --- | --- | --- | --- | --- |
| Globalstar | Globalstar Partnership | International long-haul communication, vehicle op., traveling national and international | LEO satellite; digital; wireless telecom; extend terrestrial cellular | Digital voice and data; 2400–9600 bps | CDMA/FDMA |
| Iridium | Iridium | Enhanced mobile satellite systems market focus | 66 LEO satellites; provide use of portable satellite phones | 2.4 kbps–4.8 kbps | TDMA/FDMA |
| AMSC | American Mobile Satellite Corp. | United States, Puerto Rico, Virgin Islands, 200 miles of coastal waters; land mobile, fixed site, and aeronautical users | Mobile communications for voice, data, digital broadcast dispatch | 1.2 kbps–4.8 kbps | FDMA |
| Ellipso | Ellipsat | Global mobile communication market | LEO; global voice and data services for access to unserved and remote areas | 300–9600 bps | CDMA |
| Inmarsat | International Maritime Satellite Org. | Global personal mobile satellite communication | MEO; satellite communication for commercial emergency and safety app. | Primary modes Mode “A” - 2.4 kbps–9.6 kbps Mode “B” - Digital 0 to 4.8 kpbs Mode “M4” - ISDN, 0 to 128 kbps | Mode “A” – SCPC uplink TDM, downlink TDMA |
| ORBCOMM | Orbital Communications Corp. | Real-time mobile two-way data and messaging services worldwide for U.S. Armed Forces, transportation, utility, oil, and gas | LEO; provides wireless e-mail, fax, and GPS; small fleet and high value asset location and alarm monitoring use | Iplink 2.4 kbps; downlink 4.8 kbps | TBS |
| Odyssey | Odyssey Worldwide Services | Global coverage | MEO; wireless personal communication | 4.8 kbps | CDMA |

Table C-2.  Public/Commercial Networks

| Acronym | Full Name | Protocol Authority | Used By | Purpose | Security | User Bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
| Internet | Internet | Internet Engineering Steering Group (IESG) | Worldwide | Voice, video, and data applications | N/A | 300 bps–1.544Mbps |
| ATM | Asynchronous Transfer Mode | ATM Forum | Data carriers | Wide area networking | data privacy | 2.4 Gbps typical |
| SONET/SDH | Synchronous Optical Network Synchronous Digital Hierarchy | Bellcore/CCITT; ANSI T1.105, T1.107; ANSI T1.106 and ANSI T1.117, ITU-T | United States, Europe, Japan | Transport of many digital signals with different capacities | N/A | 51.48 Mbps up to 2405.376 Mbps |
| PSTN | Public Switched Telephone Network | ITU-14 pending | Worldwide | Provides a wide variety of voice and data services | N/A | 9.6 kbps–155 Mbps |

Table C-3.  DoD Networks

| Acronym | Full Name | Managed By | Used By | Purpose | Security | Bandwidth (for User Services) | Nature of User Access to Network |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SIPRNet | Secret Internet Protocol Router Network | DISA | U.S. DoD | Transport of classified and mission-critical data for DoD users | Network operates system high Secret; links encrypted | 64 kbps–1.544 Mbps | Direct network access |
| ATDNet | Advanced Technology Demonstration Network | DISA | TBS | Research network with various nodes | TBS | TBS | TBS |
| NIPRNet | Nonclassified Internet Protocol Router Network | DISA | U.S. DoD | Transport of official data for DoD users | Network operates FOUO. Direct connected to the Internet | Up to T-3 rates | Direct network access and mobile dialin support |
| DISN | Defense Information System Network (Superset of SIPRNet and NIPRNet) | DISA | Global DoD community | Primarily data transport system for DII, including some voice and video | Multiple networks running system high with cryptographic separation via trunk encryptors | Currently as high as DS3 with pressure to increase capacity | Direct and remote dial-in |
| S-ATM (DAS-C) | DISN ATM Wide Area Network | DISA | DoD community | Secret ATM network; transport of high-speed/bandwidth mission-critical info. | Encryption | T1 up to OC-3 rates | Via agency network(s) |
| U-ATM (DAWN; DAS-U) | DISN ATM Wide Area Network | DISA | DoD community |  | N/A | T1 up to OC-3 rates | Via agency network(s) |
| JWICS | Joint Worldwide Intelligence Communications System | DIA | DoD community and civilian government agencies | Data and video transport; video teleconferencing at SCI level | Link encryption | 56 kbps up to T1 rates | Through agency network or fixed facility |
| ATMAII | TBS | TBS | TBS | TBS | TBS | TBS | TBS |
| DSINet | Defense Simulation Internet | TBS | TBS | TBS | TBS | TBS | TBS |
| DREN | Defense Research and Engineering Network | DARPA and DISA | TBS | TBS | TBS | 155–622 Mbps | TBS |
| DRSN | Defense Red Switch Network | DISA | Global DoD community | Primarily classified voice system | Trunk encryption | TBS | Phone set |
| Red IDNX | Red Integrated Digital Network Exchange | USAF | DISN | Data aggregation to achieve bandwidth efficiencies | Trunk encryption | T1 and below | Integrated into transport system |
| Black IDNX | TBS | TBS | TBS | TBS | TBS | TBS | TBS |
| MILSTAR | Military Strategic Tactical Relay System | Operated by Air Force Space Command; owned by U.S. Space Command | U.S. military | Emergency action message dissemination; provides tactical survivable communication, including MSE range ext. | Encryption | 75 bps–2.4 kbps; medium data rate capability 4.8 kbps–1.544 Mbps | MILSTAR terminal |

Table C-4.  Networking Technologies

| Acronym | Full Name | Protocol Authority | Used By | Purpose | Security | Bandwidth (for User Services) | Multiple Access Method |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ISDN | Integrated Services Digital Network | ITU-T, Q.921, Q.931 ANSI T1.601, T1.408; and Bellcore SR 3875 | Worldwide | Network browsing, transferring data, remote access | N/A | 64000 bps– 1.544 Mbps | N/A |
| SONET/SDH | Synchronous Optical Network/ Synchronous Digital Hierarchy | Bellcore/CCITT; ANSI T1.105, ANSI T1.106 and ANSI T1.117, ITU-T | United States Europe Japan | TBS | TBS | 51.48 Mbps up to 2405.376 Mbps | N/A |
| ATM | Asynchronous Transfer Mode | ATM Forum | Data carriers | Wide area networking; multimedia and video applications | Data privacy | 2.4 Gbps typical | N/A |
| AMPS | Advanced Mobile Phone Service | EIA/TIA-553 U.S. | United States | Analog voice | N/A | Up to 13 kbps actual data throughput using CDMA tech. | FDMA |
| PCS | Personal Communications Service | TIA IS-136A/137A/138A; J-Stds-009/010/011 ETSI PCS 1900, (PCN) DCS 1800 | United States Europe | Voice and data services; paging-type services; mobile communications | Authentication and Privacy | 7.95 kbps–13 kbps | TDMA FDMA |
| GSM | Global System for Mobile Communication | European Telecommunications Standardization Institute (ETSI) | Europe | Digital voice, data, SMS | Authentication and Privacy | 9,600 bps | TDMA FDMA |
| DCS | Digital Cellular System | TIA IS-95; IS-136A (D-Amps) | United States | Digital voice, data | Authentication | 7.9 kbps | TDMA FDMA |
| Frame Relay | Frame Relay | Frame Relay Forum (NNI std)-future | United States; corporations | Data communications; some voice and video | N/A | 56 kbps– 1.544 Mbps | TBS |
| SMDS | Switched Multi-megabit Data Service | Compatible with IEEE 802.6 MAN and B-ISDN | Local exchange carrier customers requiring large communications pipeline | Large data transfers; video, graphics, CAD/CAM, x-rays | N/A | 1.544 Mbps–45 Mbps | TBS |

This page intentionally left blank.